# OpenLCB CDI Viewer

/* Node ID

Node Name	Address = 0; 0x0	(Max: 62 chars)

Node Description	Address = 62; 0x3E	(Max: 63 chars)

Reset Control		Address = 125; 0x7D	(1 byte)

Attributes

Number of Strings	Address = 126; 0x7E	(1 bytes, Max: 255)

Maximum Luminosity	Address = 127; 0x7F	(1 bytes, Max: 255)

Full Luminosity On	Address = 128; 0x80	(8 bytes, Event ID)

Minimum Luminosity	Address = 136; 0x88	(1 bytes, Max: 255)

Low Luminosity On	Address = 137; 0x89	(8 bytes, Event ID)

Effect Cycle Frequency	Address = 145; 0x91	(1 bytes, Max: 255)

Group Events

Use these to control groupings of individual LEDs across all active strings

All On	Address = 146; 0x92	(8 bytes, Event ID)

All OFF	Address = 154; 0x9A	(8 bytes, Event ID)

Street On	Address = 162; 0xA2	(8 bytes, Event ID)

Street OFF	Address = 170; 0xAA	(8 bytes, Event ID)

Exterior On	Address = 178; 0xB2	(8 bytes, Event ID)

Exterior OFF	Address = 186; 0xBA	(8 bytes, Event ID)

Business On	Address = 194; 0xC2	(8 bytes, Event ID)

Business OFF	Address = 202; 0xCA	(8 bytes, Event ID)

Residential On	Address = 210; 0xD2	(8 bytes, Event ID)

Residential OFF	Address = 218; 0xDA	(8 bytes, Event ID)

Secondary On	Address = 226; 0xE2	(8 bytes, Event ID)

Secondary OFF	Address = 234; 0xEA	(8 bytes, Event ID)

Strings

Parameters for the lighting area NeoPixel strings

String 1	Address = 242; 0xF2

String 2	Address = 1100; 0x44C

String 3	Address = 1958; 0x7A6

String 4	Address = 2816; 0xB00

[maybe make these offsets from the starting address of the String #]

Description	Address = 242; 0xF2	(Max: 21 chars)

Number of heads	Address = 263; 0x107	(1 bytes, Max: 255)

All On	Address = 264; 0x108	(8 bytes, Event ID)

All Off	Address = 272; 0x110	(8 bytes, Event ID)

All Toggle	Address = 280; 0x118	(8 bytes, Event ID)

Effects On	Address = 288; 0x120	(8 bytes, Event ID)

Effects Off	Address = 296; 0x128	(8 bytes, Event ID)

Effects Toggle	Address = 304; 0x130	(8 bytes, Event ID)

Dimmer Toggle	Address = 312; 0x138	(8 bytes, Event ID)

[maybe make these offsets from the starting address of the String #]

Neo-Pixels	1	Address = 320; 0x140

…..

Neo-Pixels	20	Address = 1061; 0x425

[maybe make these offsets from the starting address of the Neo-Pixels]

Description	Address = 320; 0x140	(Max: 21 chars)

Red Lead

Intensity	Address = 341; 0x155	(1 bytes, Max: 255)

Cycles On	Address = 342; 0x156	(1 bytes, Max: 255)

Cycles Off	Address = 343; 0x157	(1 bytes, Max: 255)

Starting Cycle	Address = 344; 0x158	(1 byte, Default: 0)

Effect	Address = 345; 0x159	(1 byte, Default: 1)

Group	Address = 346; 0x15A	(1 byte, Default: 0)

Green Lead

Intensity	Address = 347; 0x15B	(1 bytes, Max: 255)

Cycles On	Address = 348; 0x15C	(1 bytes, Max: 255)

Cycles Off	Address = 349; 0x15D	(1 bytes, Max: 255)

Starting Cycle	Address = 350; 0x15E	(Default: 0)

Effect	Address = 351; 0x15F	(Default: 1)

Blue Lead

Group	Address = 352; 0x160	(Default: 0)

Intensity	Address = 353; 0x161	(1 bytes, Max: 255)

Cycles On	Address = 354; 0x162	(1 bytes, Max: 255)

Cycles Off	Address = 355; 0x163	(1 bytes, Max: 255)

Starting Cycle	Address = 356; 0x164	(Default: 0)

Effect	Address = 357; 0x165	(Default: 1)

Group	Address = 358; 0x166	(Default: 0)

*/